Assignment 1
Esraa Ashraf Gamal
Group: S26-B10-FPGA-G10

Comparison of Verilog Abstraction Levels

| Level | Main Focus | Keywords / Constructs | Usage | Synthesizable |
| --- | --- | --- | --- | --- |
| Behavioral | Describes what the circuit does | always, initial, if, case | Algorithm description and complex logic | Yes (if written correctly) |
| Dataflow | Describes data movement using equations | assign, operators | Combinational circuits | Yes |
| Gate | Describes logic gates directly | and, or, xor, not | Small digital circuits | Yes |
| Structural | Describes hardware using modules and connections | module instantiation | Hierarchical designs | Yes |

1. Gate-Level Full Adder

full_adder_gate.v

module full_adder_gate(

input A,

input B,

input Cin,

output Sum,

output Cout);

wire x1;

wire a1;

wire a2;

xor(x1, A, B);

xor(Sum, x1, Cin);

and(a1, A, B);

and(a2, x1, Cin);

or(Cout, a1, a2);

endmodule

2. Structural Full Adder

full_adder_structural.v

module half_adder(

input A,

input B,

output Sum,

output Carry);

xor(Sum, A, B);

and(Carry, A, B);

endmodule

module full_adder_structural(

input A,

input B,

input Cin,

output Sum,

output Cout);

wire s1;

wire c1;

wire c2;

half_adder HA1(

A,

B,

s1,

c1);

half_adder HA2(

s1,

Cin,

Sum,

c2);

or(

Cout,

c1,

c2);

endmodule

3. Behavioral Full Adder

full_adder_behavioral.v

module full_adder_behavioral(

input A,

input B,

input Cin,

output Sum,

output Cout);

assign {Cout, Sum} = A + B + Cin;

endmodule

Testbench

tb.v

module tb;
reg A;
reg B;
reg Cin;

wire Sum_gate;

wire Cout_gate;

wire Sum_struct;

wire Cout_struct;

wire Sum_beh;

wire Cout_beh;

full_adder_gate G1(

A,

B,

Cin,

Sum_gate,

Cout_gate);

full_adder_structural S1(

A,

B,

Cin,

Sum_struct,

Cout_struct);

full_adder_behavioral B1(

A,

B,

Cin,

Sum_beh,

Cout_beh);

initial begin

A=0; B=0; Cin=0;

#10;

A=0; B=0; Cin=1;

#10;

A=0; B=1; Cin=0;

#10;

A=0; B=1; Cin=1;

#10;

A=1; B=0; Cin=0;

#10;

A=1; B=0; Cin=1;

#10;

A=1; B=1; Cin=0;

#10;

A=1; B=1; Cin=1;

#10;

$finish;

end

initial begin

$monitor("A=%b B=%b Cin=%b | Gate=%b%b Structural=%b%b Behavioral=%b%b",

A,B,Cin,Cout_gate,Sum_gate,Cout_struct,Sum_struct,Cout_beh,Sum_beh);

end

endmodule